Mbus test setup

Two meters 280W-D and 280T connected to Mbus concentrator input

Detailed wiring connection to meters

Concentrator

AC power in                                       COM port to PC Mbus I/O from meters

M Bus Sheet V1.6 showing meters from COM2

Chart to determine what 280 meter you have

| Type | No. of Wires | Colors |
| --- | --- | --- |
| Encoder | 3 | Black, Red, Yellow |
| MBus | 2 | Red, Black |
| Pulse (w/o battery) | 2 | Yellow, White |
| Pulse (w/ battery) | 4 | Yellow, White, Red, Black |
| Modbus | 4 | Red, Black, Blue, Yellow |
| Modbus + 4-20mA | 6 | Red, Orange, White, Black, Yellow, Green |

Encoder (3 wires)

Black     Gnd

Red          Vdc

Yellow   D+

MBus  (2 wires)

Red          MBus +

Black      MBus –

Pulse (w/o battery)   (2 wires)

Yellow   B-

White     A+

Pulse (w/ battery)   (4 wires)

Red           Vdc

Black        GND

Yellow    B-

White      A+

Modbus (4 wires)

Red           Vdc

Black       GND

Blue         A+

Yellow    B-

Modbus + 4-20ma  (6 wires)

Red           Vdc

Orange   Ao+

White      Ao-

Black       GND

Yellow    485B

Green       485A